For re-programming, the code on the Invictus lock

Default code is 0000

STEP 1:

Press Camec logo twice >(LED indicates yellow) Press zero 6 times (000000) > Press Camec logo once (LED indicates blue )

STEP 2:

Press 0,1 and Camec logo (LED indicates Magenta)

STEP3:

Enter new 4-digit code > press Camec logo (LED indicates Teal)

Re-enter new code > press Camec logo (LED indicates Green)

New code is saved.

NOTE:

Steps should be carried out continuous before the LED goes out. If LED goes out, start from scratch.